LOOGOOTEE MFG. — TRAINING SIMULATION MICRO850 PROGRAM SPECIFICATION Palletizer and Bulk Bin Staging · Site 1 · Area: Palletizing Prepared by: Jeremy Field · Senior Automation Technician Loogootee Mfg., Inc. · Loogootee, Illinois · March 2026 Document Version 1.0 · Training Simulation — Not for Distribution

TRAINING SIMULATION NOTICE This document specifies a simulated PLC program for a fictional company. All process values, timing parameters, and equipment references are illustrative.

1.  Machine Overview

The Micro850 simulates the palletizer and bulk bin staging operation at the end of the Site 1 production line. It receives finished, inspected parts from the inspection and sorting station, accumulates them into bulk bins to a configured target count, manages lot numbering, and signals Site 2 when a completed bin is ready for transfer to the packaging operation.

The machine operates fully autonomously in Auto mode. Bin arrivals are simulated by a timer-based event rather than a physical sensor, allowing continuous unattended operation for data generation purposes. Fault events are simulated on timer and counter-based schedules with automatic 20-second recovery, modeling realistic operator response and intervention.

| Controller | Allen-Bradley Micro850 — 2080-LC50-48QBB |
| --- | --- |
| Software | Connected Components Workbench (CCW) |
| Site / Area | Site 1 — Palletizing |
| UNS Namespace | Site1_Palletizing |
| Machine Function | Palletizer / bulk bin fill and staging — end of Site 1 line |
| Upstream | Receives part count signal from Inspection station via UNS |
| Downstream | Publishes Site1_Palletizing_Transfer_Request and Site1_Palletizing_Lot_Number to UNS — received by Wago PFC200 |
| Shift Reset | CLR instructions triggered by Node-RED shift reset topic via EPIC |
| Tag Standard | ISA-95 Snake_Case — all tags conform to project naming standard |

2.  I/O Summary

Physical I/O is limited to the operator control station. No field device I/O is used for cycle control — all cycle triggers are timer-based internal events. The bin-in-position signal that would exist on a real palletizer is replaced by the Site1_Palletizing_Bin_Arrival_Timer done bit in Routine 2.

| Point | Type | Signal | Tag Name | Description |
| --- | --- | --- | --- | --- |
| I0 | DI | 24VDC | Site1_Palletizing_Op_Cycle_Start | Momentary PB green — manual cycle start in Manual mode |
| I1 | DI | 24VDC | Site1_Palletizing_Op_Cycle_Stop | Momentary PB red — stop cycle |
| I2 | DI | 24VDC | Site1_Palletizing_Op_Auto_Select | Selector switch — Auto / Manual mode |
| I3 | DI | 24VDC | Site1_Palletizing_Op_Manual_Select | Momentary PB yellow — manual fault reset |
| I4 | DI | 24VDC | Site1_Palletizing_Op_Fault_Reset | Maintained PB red mushroom — E-Stop simulation |
| I5 | DI | 24VDC | Site1_Palletizing_Op_Estop_Sim | Maintained PB red mushroom — E-Stop simulation |
| O0 | DO | 24VDC | Site1_Palletizing_Light_Running | Pilot light green — machine running |
| O1 | DO | 24VDC | Site1_Palletizing_Light_Faulted | Pilot light red — active fault |
| O2 | DO | 24VDC | Site1_Palletizing_Light_Auto | Pilot light amber — Auto mode active |
| CH0 | AI | 0–10V | Site1_Palletizing_Parts_Per_Bin_Ref | Potentiometer — adjustable parts per bin target |
| CH1 | AI | 4–20mA | Site1_Palletizing_Fill_Rate_Ref | Potentiometer — adjustable fill rate reference |

3.  Program Structure

The program is organized into five routines. All routines are called from Routine 1 — Main on every scan. Output coils are energized only in Routine 5. No output is written outside of Routine 5.

| Routine | Name | Purpose |
| --- | --- | --- |
| R01 | Main | Program entry point. Mode control. Shift reset handler. Routine call sequencing. |
| R02 | Cycle_Control | State machine. Bin arrival simulation. Loading cycle. Bin complete logic. |
| R03 | Fault_Alarms | Fault simulation — timer and counter based. Auto recovery timer. Downtime logging. |
| R04 | Production_Data | OEE calculations. Shift accumulators. Lot number generation. Transfer handshake. |
| R05 | Outputs | All physical output coils. Energized here only. |

4.  Routine Specifications

R01 — Main

Program entry point. Executes every scan. Calls all other routines in sequence. Contains mode control logic and shift reset handler. No process logic lives in this routine.

Mode Control

Two discrete inputs control mode selection. Site1_Palletizing_Op_Auto_Select sets Site1_Palletizing_Machine_Auto or Site1_Palletizing_Machine_Manual bit. Mode change only permitted when machine is in IDLE state. Mode change while running or faulted is ignored.

Shift Reset Handler

Node-RED publishes shift reset trigger to MQTT broker at each shift start time. EPIC subscribes to the topic and writes a value to a designated Micro850 tag via EtherNet/IP. On receipt, a one-shot instruction fires CLR instructions against all shift accumulator .ACC values simultaneously. Site1_Palletizing_Shift_Number tag updates to the new shift value (1, 2, or 3) published by Node-RED.

Shift reset sequence (one-shot on trigger received): CLR Site1_Palletizing_Parts_Shift.ACC CLR Site1_Palletizing_Good_Parts_Shift.ACC CLR Site1_Palletizing_Reject_Count_Shift.ACC CLR Site1_Palletizing_Scrap_Count_Shift.ACC CLR Site1_Palletizing_Fault_Count_Shift.ACC CLR Site1_Palletizing_Pallet_Bin_Count_Shift.ACC CLR Site1_Palletizing_Runtime_Sec.ACC CLR Site1_Palletizing_Downtime_Sec.ACC MOV Site1_Palletizing_Shift_Number_From_EPIC --> Site1_Palletizing_Shift_Number

Routine Call Sequence

JSR R02_Cycle_Control JSR R03_Fault_Alarms JSR R04_Production_Data JSR R05_Outputs

R02 — Cycle Control

Core machine state machine. Controls the simulated bin arrival, loading cycle, and bin completion sequence. All cycle logic is contained here. State is encoded in a single integer tag — Site1_Palletizing_Machine_State — that all other routines can read for context.

State Machine Definition

| State | Value | Description |
| --- | --- | --- |
| IDLE | 0 | Machine ready. Waiting for bin arrival event. Site1_Palletizing_Bin_Arrival_Timer running. |
| LOADING | 1 | Bin arrival event received. Loading cycle active. Site1_Palletizing_Parts_Load_Timer running. Site1_Palletizing_Parts_Counter incrementing. |
| BIN_COMPLETE | 2 | Site1_Palletizing_Parts_Counter reached target. One scan state — triggers lot close and transfer request. Transitions to IDLE. |
| FAULTED | 3 | Active fault. Machine stopped. Site1_Palletizing_Fault_Recovery_Timer running. Auto-recovers on timer done. |
| MANUAL | 4 | Manual mode active. Automatic cycle suspended. Operator controls cycle via Site1_Palletizing_Op_Cycle_Start input. |

Bin Arrival Simulation

A free-running timer — Site1_Palletizing_Bin_Arrival_Timer — replaces the physical bin-in-position sensor. When the timer done bit fires, a one-shot triggers the IDLE to LOADING state transition. The timer resets and begins counting the next arrival interval immediately.

Pseudo-random variance is applied to the timer preset on each reset. A MVM (masked move) instruction reads the lower 4 bits of a free-running millisecond counter and adds the result to a base preset value. This produces arrival intervals that vary by plus or minus a few seconds around the base value, preventing perfectly regular data in the historian.

Base arrival interval: 45 seconds (configurable) Variance range: +/- 8 seconds via MVM on ms counter bits Effective interval range: 37 to 53 seconds On timer DN bit (one-shot): MVM Site1_Palletizing_Free_Run_Ms_Timer.ACC mask:0x000F --> Site1_Palletizing_Arrival_Variance ADD Site1_Palletizing_Arrival_Base_Preset Site1_Palletizing_Arrival_Variance --> Site1_Palletizing_Bin_Arrival_Timer.PRE RES Site1_Palletizing_Bin_Arrival_Timer MOV 1 --> Site1_Palletizing_Machine_State (transition to LOADING)

Loading Cycle

In LOADING state, Site1_Palletizing_Parts_Load_Timer runs continuously. On each timer done bit (one-shot), Site1_Palletizing_Parts_Counter increments by a small batch value simulating parts dropping into the bin. A compare instruction checks Site1_Palletizing_Parts_Counter against Site1_Palletizing_Parts_Per_Bin_Target on every scan. When equal or greater, the machine transitions to BIN_COMPLETE state.

Site1_Palletizing_Parts_Load_Timer preset: 2 seconds (one increment per 2 seconds) Increment per timer DN: Site1_Palletizing_Parts_Counter += Site1_Palletizing_Load_Batch_Size (default: 5 parts) Site1_Palletizing_Parts_Per_Bin_Target: Read from analog input — scaled 100 to 300 parts On timer DN (one-shot): ADD Site1_Palletizing_Parts_Counter Site1_Palletizing_Load_Batch_Size --> Site1_Palletizing_Parts_Counter Compare on every scan: GEQ Site1_Palletizing_Parts_Counter Site1_Palletizing_Parts_Per_Bin_Target --> transition to BIN_COMPLETE

Bin Complete

BIN_COMPLETE is a one-scan state. On entry, it sets the Site1_Palletizing_Pallet_Complete one-shot bit, triggers lot close logic in R04, latches Site1_Palletizing_Transfer_Request, and resets Site1_Palletizing_Parts_Counter. The state machine immediately transitions back to IDLE to begin the next bin arrival cycle.

R03 — Fault and Alarm Management

Two independent simulated fault sources. Both faults transition the state machine to FAULTED state, log the event, and start the auto-recovery timer. The 20-second recovery window simulates realistic operator response — operator observes the alarm, walks to the machine, physically corrects the condition, and presses cycle start.

Fault 01 — Jam / Bin Misalign (Timer Based)

A free-running timer generates periodic fault events independent of production count. Pseudo-random variance via MVM on free-running counter bits varies the interval so faults do not occur on a predictable schedule. Simulates nuisance jams or bin positioning errors common on vibratory feed and palletizing equipment.

Base fault interval: 180 seconds (3 minutes) Variance: +/- 30 seconds via MVM on free-running counter Effective range: 150 to 210 seconds between Fault 01 events On Site1_Palletizing_Fault_01_Timer DN (one-shot): MOV 1 --> Site1_Palletizing_Fault_Active MOV 1 --> Site1_Palletizing_Fault_Code MOV "JAM / BIN MISALIGN" --> Site1_Palletizing_Fault_Description MOV 3 --> Site1_Palletizing_Machine_State (transition to FAULTED) CTU Site1_Palletizing_Fault_Count_Shift capture current time --> Site1_Palletizing_Downtime_Start_Time RES + reload Site1_Palletizing_Fault_01_Timer with new random preset

Fault 02 — Mechanical Overload Simulation (Counter / Compare Based)

A counter increments on every bin complete event. A compare instruction checks the counter against a target value. When the counter reaches the target, Fault 02 fires. On each fault clear, a new target is loaded using MVM on free-running counter bits to randomize the next interval. Simulates wear-based or load-based fault patterns — equipment that faults more frequently as production accumulates.

Base bin count between faults: 15 bins Variance: +/- 5 bins via MVM on free-running counter Effective range: 10 to 20 bins between Fault 02 events On bin complete: CTU Site1_Palletizing_Fault_02_Counter Compare every scan: GEQ Site1_Palletizing_Fault_02_Counter.ACC Site1_Palletizing_Fault_02_Target --> Fault 02 trigger On Fault 02 trigger (one-shot): MOV 1 --> Site1_Palletizing_Fault_Active MOV 2 --> Site1_Palletizing_Fault_Code MOV "MECHANICAL OVERLOAD SIM" --> Site1_Palletizing_Fault_Description MOV 3 --> Site1_Palletizing_Machine_State (transition to FAULTED) CTU Site1_Palletizing_Fault_Count_Shift capture time --> Site1_Palletizing_Downtime_Start_Time RES Site1_Palletizing_Fault_02_Counter MVM + ADD --> new Site1_Palletizing_Fault_02_Target

Auto Recovery Timer

Site1_Palletizing_Fault_Recovery_Timer starts when Site1_Palletizing_Machine_State transitions to FAULTED. Preset: 20 seconds.

Site1_Palletizing_Fault_Recovery_Timer preset: 20 seconds On Site1_Palletizing_Fault_Recovery_Timer DN (one-shot): MOV 0 --> Site1_Palletizing_Fault_Active MOV Site1_Palletizing_Fault_Code --> Site1_Palletizing_Last_Fault_Code (log before clear) MOV 0 --> Site1_Palletizing_Fault_Code MOV "" --> Site1_Palletizing_Fault_Description SUB current_time Site1_Palletizing_Downtime_Start_Time --> Site1_Palletizing_Downtime_Duration_Sec MOV 0 --> Site1_Palletizing_Machine_State (transition to IDLE) (Site1_Palletizing_Bin_Arrival_Timer reset fires in R02 — cycle resumes automatically)

R04 — Production Data and OEE

All accumulation, calculation, and lot management logic. Tags calculated here are the primary data published to the UNS. All shift-accumulating tags have corresponding CLR instructions in R01 shift reset handler.

Runtime and Downtime Accumulation

Site1_Palletizing_Runtime_Sec accumulates when: Site1_Palletizing_Machine_State = LOADING (1) Site1_Palletizing_Downtime_Sec accumulates when: Site1_Palletizing_Machine_State = FAULTED (3) Both use TON timers with .ACC read each scan Site1_Palletizing_Planned_Production_Time_Sec: Set at shift start = shift duration in seconds (28800 = 8hr)

OEE Calculations

Site1_Palletizing_Availability_Pct = (Runtime_Sec / Planned_Production_Time_Sec) * 100 Site1_Palletizing_Performance_Pct = (Ideal_Cycle_Time_Sec / Cycle_Time_Sec) * 100 Site1_Palletizing_Quality_Pct = (Good_Parts_Shift / Parts_Shift) * 100 Site1_Palletizing_OEE_Pct = (Availability_Pct * Performance_Pct * Quality_Pct) / 10000 Site1_Palletizing_Ideal_Cycle_Time_Sec: Constant — set per machine spec (base bin fill time) Site1_Palletizing_Cycle_Time_Sec: Measured — Parts_Load_Timer.ACC at bin complete Fault 02 (overload) increments Reject_Count_Shift by fixed value on each event Scrap_Count_Shift increments by 1 on each fault event (any fault) Good_Parts_Shift = Parts_Shift - Reject_Count_Shift - Scrap_Count_Shift

Parts Per Hour Calculation

Calculated every 60 seconds via TON timer done bit Parts_Per_Hour_Actual = (Parts_Shift / Runtime_Sec) * 3600 Protected against divide by zero — only calculates when Runtime_Sec > 0

Lot Number Generation

Lot number is constructed on each bin complete event. Two components concatenated into a STRING tag: date portion (YYYYMMDD) passed in from EPIC system clock, and a 5-digit zero-padded sequence number maintained as a DINT counter that increments on every bin complete and resets at midnight via Node-RED shift reset mechanism.

Site1_Palletizing_Lot_Sequence_Counter: CTU on BIN_COMPLETE state entry (one-shot) Site1_Palletizing_Lot_Date_String: Passed from EPIC via EtherNet/IP tag write — updates at midnight Lot number construction on bin complete: Format Lot_Sequence_Counter as zero-padded 5-digit string --> Lot_Seq_String CONCAT Lot_Date_String "-" Lot_Seq_String --> Site1_Palletizing_Lot_Number Lot quantity variance via masked move: MVM Free_Run_Ms_Timer.ACC mask:0x001F --> Lot_Qty_Variance ADD Parts_Per_Bin_Base Lot_Qty_Variance --> Lot_Part_Count Example output: 20260328-00047

Transfer Request Handshake

Site1_Palletizing_Transfer_Request latches TRUE on bin complete. It remains TRUE until EPIC acknowledges receipt via a corresponding handshake tag written back to the Micro850. On acknowledgement, Site1_Palletizing_Transfer_Request clears and Site1_Palletizing_Pallet_Bins_Staged increments. This ensures no transfer event is lost if the EPIC or Node-RED is momentarily busy.

On BIN_COMPLETE (one-shot): MOV 1 --> Site1_Palletizing_Transfer_Request (latches — published to UNS) MOV 1 --> Site1_Palletizing_Lot_Complete (one-shot bit) On EPIC acknowledgement received: MOV 0 --> Site1_Palletizing_Transfer_Request (clears latch) CTU Site1_Palletizing_Pallet_Bins_Staged (increments staged bin count)

R05 — Outputs

All physical output coils are energized here and only here. No output instruction appears in any other routine. This ensures complete visibility of all output conditions in one location and prevents conflicting output logic across routines.

O0 — Site1_Palletizing_Light_Running: Energized when: Site1_Palletizing_Machine_State = LOADING (1) O1 — Site1_Palletizing_Light_Faulted: Energized when: Site1_Palletizing_Fault_Active = 1 O2 — Site1_Palletizing_Light_Auto: Energized when: Site1_Palletizing_Machine_Auto = 1

5.  Tag Summary — UNS Published Tags

All tags below are published to the Unified Namespace via EPIC Node-RED flows. Full tag names follow the ISA-95 naming convention — prefix Site1_Palletizing_ applies to all tags in this program. These tags correspond directly to the subscription register Section 5.8.

Machine Status

| Tag Name | Type | Unit | Description / Behavior |
| --- | --- | --- | --- |
| Site1_Palletizing_Machine_Running | BOOL | -- | TRUE when Site1_Palletizing_Machine_State = LOADING |
| Site1_Palletizing_Machine_Stopped | BOOL | -- | TRUE when Site1_Palletizing_Machine_State = IDLE and not faulted |
| Site1_Palletizing_Machine_Faulted | BOOL | -- | TRUE when Site1_Palletizing_Machine_State = FAULTED |
| Site1_Palletizing_Machine_Auto | BOOL | -- | TRUE when Auto mode active |
| Site1_Palletizing_Machine_Manual | BOOL | -- | TRUE when Manual mode active |
| Site1_Palletizing_Estop_Active | BOOL | -- | TRUE when Site1_Palletizing_Op_Estop_Sim input is active |
| Site1_Palletizing_Machine_State | INT | -- | Current state machine value 0–4 |

OEE — Availability

| Tag Name | Type | Unit | Description / Behavior |
| --- | --- | --- | --- |
| Site1_Palletizing_Planned_Production_Time_Sec | DINT | Sec | Shift duration in seconds — set at shift start |
| Site1_Palletizing_Runtime_Sec | DINT | Sec | Accumulated loading time this shift |
| Site1_Palletizing_Downtime_Sec | DINT | Sec | Accumulated fault time this shift |
| Site1_Palletizing_Availability_Pct | REAL | Pct | Runtime / Planned × 100 |

OEE — Performance

| Tag Name | Type | Unit | Description / Behavior |
| --- | --- | --- | --- |
| Site1_Palletizing_Cycle_Time_Sec | REAL | Sec | Measured bin fill time — last completed cycle |
| Site1_Palletizing_Ideal_Cycle_Time_Sec | REAL | Sec | Target bin fill time at rated speed — constant |
| Site1_Palletizing_Parts_Per_Hour_Actual | REAL | Pph | Calculated actual parts per hour |
| Site1_Palletizing_Parts_Per_Hour_Target | REAL | Pph | Target parts per hour at ideal cycle time |
| Site1_Palletizing_Performance_Pct | REAL | Pct | Ideal / Actual cycle time × 100 |

OEE — Quality

| Tag Name | Type | Unit | Description / Behavior |
| --- | --- | --- | --- |
| Site1_Palletizing_Parts_Total | DINT | Count | Cumulative all-time part count — never resets |
| Site1_Palletizing_Parts_Shift | DINT | Count | Parts this shift — CLR on shift reset |
| Site1_Palletizing_Good_Parts_Shift | DINT | Count | Parts_Shift minus rejects and scrap |
| Site1_Palletizing_Reject_Count_Shift | DINT | Count | Reject increment on Fault 02 event |
| Site1_Palletizing_Scrap_Count_Shift | DINT | Count | Scrap increment on any fault event |
| Site1_Palletizing_Reject_Pct | REAL | Pct | Reject_Count_Shift / Parts_Shift × 100 |
| Site1_Palletizing_Scrap_Pct | REAL | Pct | Scrap_Count_Shift / Parts_Shift × 100 |
| Site1_Palletizing_Quality_Pct | REAL | Pct | Good_Parts_Shift / Parts_Shift × 100 |
| Site1_Palletizing_OEE_Pct | REAL | Pct | Availability × Performance × Quality / 10000 |

Fault and Downtime

| Tag Name | Type | Unit | Description / Behavior |
| --- | --- | --- | --- |
| Site1_Palletizing_Fault_Active | BOOL | -- | TRUE when machine is in FAULTED state |
| Site1_Palletizing_Fault_Code | INT | -- | 1 = Jam/Misalign 2 = Overload Sim 0 = No fault |
| Site1_Palletizing_Fault_Description | STRING | -- | Plain text — current active fault |
| Site1_Palletizing_Fault_Count_Shift | INT | Count | Fault events this shift |
| Site1_Palletizing_Downtime_Start_Time | DINT | -- | Timestamp of most recent fault event |
| Site1_Palletizing_Downtime_Duration_Sec | DINT | Sec | Duration of most recent fault — logged on clear |
| Site1_Palletizing_Last_Fault_Code | INT | -- | Fault code of most recently cleared fault |

Palletizer Specific

| Tag Name | Type | Unit | Description / Behavior |
| --- | --- | --- | --- |
| Site1_Palletizing_Pallet_Bin_Fill_Pct | REAL | Pct | Parts_Counter / Parts_Per_Bin_Target × 100 |
| Site1_Palletizing_Pallet_Bin_Count_Shift | DINT | Count | Bins completed this shift |
| Site1_Palletizing_Pallet_Parts_Per_Bin | DINT | Count | Target parts per bin — from analog input |
| Site1_Palletizing_Pallet_Complete | BOOL | -- | One-shot TRUE on bin complete |
| Site1_Palletizing_Pallet_Bins_Staged | INT | Count | Bins staged awaiting Site 2 transfer |
| Site1_Palletizing_Lot_Number | STRING | -- | Active lot — YYYYMMDD-XXXXX format |
| Site1_Palletizing_Lot_Part_Count | DINT | Count | Total parts in active lot |
| Site1_Palletizing_Lot_Complete | BOOL | -- | One-shot TRUE on lot close |
| Site1_Palletizing_Transfer_Request | BOOL | -- | Latched TRUE on bin complete — clears on EPIC ACK |
| Site1_Palletizing_Shift_Number | INT | -- | Current shift 1 / 2 / 3 — from Node-RED via EPIC |

| DOCUMENT INFORMATION |  |
| --- | --- |
| Document Title | Micro850 Program Specification — Palletizer / Bulk Bin Staging |
| Prepared By | Jeremy Field — Senior Automation Technician |
| Version | 1.0 — Initial Release |
| Date | March 2026 |
| Status | Training Simulation — Not for Distribution |
| Repository | github.com/JeremySField/loogootee-mfg-training-simulation |
| Software | Connected Components Workbench (CCW) — Micro850 2080-LC50-48QBB |
| Related Docs | UNS Subscription Register Rev 0.1 / ISA-95 Tag Naming Standard / Project Scope of Work |

Loogootee Mfg., Inc.  ·  Training Simulation — Not for Distribution