LOOGOOTEE MFG. — TRAINING SIMULATION UNS SUBSCRIPTION REGISTER Loogootee Manufacturing, Inc. · Loogootee, Illinois Prepared by: Jeremy Field · Senior Automation Technician Loogootee Mfg., Inc. · Loogootee, Illinois · March 2026 Document Rev 0.1 · Training Simulation — Not for Distribution

TRAINING SIMULATION NOTICE This document describes the tag naming standard for a fictional company created for an IIoT training simulation. All tag names and equipment references are fabricated for educational purposes.

1.  Purpose and Scope

This document defines the UNS Subscription Register for the Loogootee Manufacturing IIoT modernization engagement. It specifies the curated set of tags that data consumers — the historian, dashboards, SCADA, and cross-site coordination logic — subscribe to from the Unified Namespace. It does not represent every tag that exists in the UNS. The Master Tag Register (a companion document) will contain the complete publish list for all machines.

This register is Rev 0.1 and will evolve as the dashboard, historian, and SCADA architecture is defined. Tags are grouped by machine and data category. Naming follows the ISA-95 Snake_Case convention defined in the project tag naming standard.

RULE This register defines what is being consumed, not everything that is available. The gap between the subscription register and the master tag register represents data published to the UNS that no current consumer subscribes to — available for future use without any change to the integration layer.

2.  Naming Convention

2.1  ISA-95 Hierarchy Structure

All tag names follow the ISA-95 equipment hierarchy. The hierarchy encodes the physical and organizational context of every data point directly in the tag name.

Enterprise └─ Loogootee_Mfg ├─ Site1_Stamping │ ├─ Stamping (MicroLogix 1400 — Stamping Press) │ ├─ Trimming (MicroLogix 1000 — Trimmer) │ ├─ Forming (Beckhoff CX5120 — High Speed Forming) │ ├─ Wash (CompactLogix L24ER — Wash Line) │ ├─ Wash_Safety (GuardLogix — Wash Line Safety) │ ├─ Inspection (S7-1200 #1 + ET200SP — Inspection & Sorting) │ ├─ Conveyor (S7-1200 #2 + ET200SP — Intra-Site Conveyor) │ ├─ Palletizing (Micro850 — Palletizer / Bulk Bin) │ └─ Supervisory (groov EPIC-PR1 — Site 1 Command) └─ Site2_Packaging └─ Packaging (Wago PFC200 — Packaging / Warehouse / Shipping)

2.2  Tag Name Formula

Every tag name follows this structure without exception:

[Site]_[Area]_[Device]_[Attribute]

| Site | Site identifier. Site1 or Site2. |
| --- | --- |
| Area | Production area. Matches the ISA-95 hierarchy area name above. |
| Device | The specific device or sub-system within the area. Optional for area-level tags. |
| Attribute | The specific data point. Includes engineering unit suffix for all analog values. |

2.3  Naming Rules

| Rule | Correct | Incorrect |
| --- | --- | --- |
| Snake_Case — capitalize each word | Parts_Stamped_Shift | parts_stamped_shift |
| Boolean — name the TRUE state | Machine_Running | Machine_Status |
| Analog — engineering unit suffix | Cycle_Time_Sec | Cycle_Time |
| Object first — noun before modifier | Motor_Left_Running | Left_Motor_Running |
| No address references | Part_Present_Sensor | Input_Slot2_Bit3 |
| Zero-padded numbers | Drive01_Speed_RPM | Drive1_Speed_RPM |
| No abbreviations unless standard | Reject_Count_Shift | Rej_Cnt_Shft |

2.4  Engineering Unit Suffixes

| Suffix | Unit | Suffix | Unit |
| --- | --- | --- | --- |
| _Sec | Seconds | _RPM | Revolutions per minute |
| _Min | Minutes | _IPM | Inches per minute |
| _Ms | Milliseconds | _PSI | Pounds per square inch |
| _F | Degrees Fahrenheit | _Lb | Pounds |
| _Pct | Percentage (0–100) | _Count | Unitless integer count |
| _Hr | Hours | _Pph | Parts per hour |

2.5  MQTT Topic Mapping

ISA-95 Snake_Case tag names map directly to Sparkplug B MQTT topic structure. No translation layer is required between the PLC data table and the Unified Namespace.

MQTT Topic: spBv1.0/[Enterprise]_[Site]_[Area]/DDATA/[EdgeNode]/[Device] Tag Metric: [Site]_[Area]_[Device]_[Attribute] Example: Topic: spBv1.0/Loogootee_Mfg_Site1_Stamping/DDATA/epic-pr1/ml1400-press01 Metric: Site1_Stamping_Press01_Parts_Stamped_Shift

3.  Equipment Register and Area Assignments

| Controller | Machine / Function | Site | Area Tag | Protocol |
| --- | --- | --- | --- | --- |
| MicroLogix 1400 | Stamping Press | Site1 | Stamping | EtherNet/IP |
| MicroLogix 1000 | Trimmer | Site1 | Trimming | DF1 via MGate |
| Beckhoff CX5120 | High Speed Forming | Site1 | Forming | OPC-UA |
| CompactLogix L24ER | Wash Line | Site1 | Wash | EtherNet/IP |
| GuardLogix | Wash Line Safety | Site1 | Wash_Safety | CIP Safety/EIP |
| S7-1200 + ET200SP | Inspection & Sorting | Site1 | Inspection | PROFINET |
| S7-1200 + ET200SP | Intra-Site Conveyor | Site1 | Conveyor | PROFINET |
| Micro850 | Palletizer / Bulk Bin | Site1 | Palletizing | EtherNet/IP |
| groov EPIC-PR1 | Site 1 Supervisory | Site1 | Supervisory | Multi-protocol |
| Wago PFC200 | Packaging / Warehouse | Site2 | Packaging | MQTT Native |

4.  Standard Tag Groups

The following tag groups are the standard data set applied to every production machine. Each group is defined once here and applied consistently across all platforms. Full tag names are constructed by prepending the Site and Area identifiers per Section 2.2.

Example: Site1_Stamping_Machine_Running, Site1_Wash_Machine_Running, Site2_Packaging_Machine_Running.

4.1  Machine Status Group — All Production Machines

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Machine_Running | BOOL | — | TRUE when machine is executing production cycles |
| Machine_Stopped | BOOL | — | TRUE when machine is intentionally stopped — not faulted |
| Machine_Faulted | BOOL | — | TRUE when an active fault is preventing operation |
| Machine_Auto | BOOL | — | TRUE when machine is in automatic mode |
| Machine_Manual | BOOL | — | TRUE when machine is in manual or jog mode |
| Estop_Active | BOOL | — | TRUE when emergency stop circuit is open |

4.2  OEE Availability Group — All Production Machines

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Planned_Production_Time_Sec | DINT | Sec | Total scheduled production time this shift |
| Runtime_Sec | DINT | Sec | Accumulated machine running time this shift |
| Downtime_Sec | DINT | Sec | Accumulated downtime this shift |
| Availability_Pct | REAL | Pct | Runtime_Sec / Planned_Production_Time_Sec × 100 |

4.3  OEE Performance Group — All Production Machines

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Cycle_Time_Sec | REAL | Sec | Actual measured cycle time — current cycle |
| Ideal_Cycle_Time_Sec | REAL | Sec | Target cycle time at rated speed — constant per machine |
| Parts_Per_Hour_Actual | REAL | Pph | Calculated actual output rate |
| Parts_Per_Hour_Target | REAL | Pph | Target output rate at ideal cycle time |
| Performance_Pct | REAL | Pct | (Ideal_Cycle_Time / Actual_Cycle_Time) × 100 |

4.4  OEE Quality Group — All Production Machines

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Parts_Total | DINT | Count | Cumulative part count — all time, never resets |
| Parts_Shift | DINT | Count | Part count this shift — resets at shift change |
| Good_Parts_Shift | DINT | Count | Parts meeting quality spec this shift |
| Reject_Count_Shift | DINT | Count | Parts rejected at this machine this shift |
| Scrap_Count_Shift | DINT | Count | Parts scrapped at this machine this shift |
| Reject_Pct | REAL | Pct | Reject_Count_Shift / Parts_Shift × 100 |
| Scrap_Pct | REAL | Pct | Scrap_Count_Shift / Parts_Shift × 100 |
| Quality_Pct | REAL | Pct | Good_Parts_Shift / Parts_Shift × 100 |

4.5  OEE Composite

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| OEE_Pct | REAL | Pct | Availability_Pct × Performance_Pct × Quality_Pct / 10000 |

4.6  Fault and Downtime Group — All Production Machines

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Fault_Active | BOOL | — | TRUE when any active fault exists |
| Fault_Code | INT | — | Active fault code — 0 when no fault |
| Fault_Description | STRING | — | Plain text fault description — current fault |
| Fault_Count_Shift | INT | Count | Number of fault events this shift |
| Downtime_Start_Time | DINT | — | Timestamp of most recent downtime event start |
| Downtime_Duration_Sec | DINT | Sec | Duration of most recent downtime event |
| Last_Fault_Code | INT | — | Fault code of most recently cleared fault |

5.  Machine-Specific Tag Groups

The following tags are unique to specific machines and supplement the standard tag groups defined in Section 4. Full tag names are constructed by prepending the Site and Area identifier per Section 2.2.

5.1  Site1_Stamping — MicroLogix 1400 Stamping Press

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Press01_Batch_Size | INT | Count | Configured batch size for current production run |
| Press01_Batch_Count | INT | Count | Completed batches this shift |
| Press01_Batch_Complete | BOOL | — | TRUE one scan when batch completes — triggers trimmer feed |
| Press01_Batch_Ready | BOOL | — | TRUE when batch is staged and ready for trimmer pickup |
| Press01_Cycles_Per_Min | REAL | — | Calculated press strokes per minute |

5.2  Site1_Trimming — MicroLogix 1000 Trimmer

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Trimmer_Bowl_Level_Pct | REAL | Pct | Vibratory feed bowl fill level — simulated |
| Trimmer_Feed_Active | BOOL | — | TRUE when feed bowl is actively feeding parts |
| Trimmer_Batch_Received | BOOL | — | TRUE when batch signal received from stamping press |
| Trimmer_Batch_Complete | BOOL | — | TRUE one scan when trimmed batch is complete |

5.3  Site1_Forming — Beckhoff CX5120 High Speed Forming

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Forming_Strokes_Per_Min | REAL | — | Current machine strokes per minute |
| Forming_Cycle_Time_Ms | REAL | Ms | Cycle time in milliseconds |
| Forming_Die_Temp_F | REAL | F | Die block temperature — simulated analog |
| Forming_Feed_Rate_IPM | REAL | IPM | Stock feed rate inches per minute |
| Forming_Tool_Wear_Pct | REAL | Pct | Simulated tool wear — increments over time |
| Forming_Batch_Complete | BOOL | — | TRUE one scan when formed batch is complete |

5.4  Site1_Wash — CompactLogix L24ER Wash Line

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Wash_Temp_F | REAL | F | Wash tank temperature |
| Wash_Rinse_Temp_F | REAL | F | Rinse tank temperature |
| Wash_Dry_Temp_F | REAL | F | Drying zone temperature |
| Wash_Chemical_Level_Pct | REAL | Pct | Wash chemical concentration — simulated |
| Wash_Water_Pressure_PSI | REAL | PSI | Wash water supply pressure |
| Wash_Batch_In_Count | DINT | Count | Cumulative batches received into wash |
| Wash_Batch_Out_Count | DINT | Count | Cumulative batches completed out of wash |
| Wash_Parts_Per_Hour | REAL | Pph | Calculated wash line throughput |

5.5  Site1_Wash_Safety — GuardLogix Safety Controller

Safety tags are read-only monitoring only. No write path exists from the integration layer to the GuardLogix. These tags reflect safety system state for visibility and alarm purposes only.

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Safety_Circuit_OK | BOOL | — | TRUE when all safety circuits are closed and healthy |
| Light_Curtain_Breach | BOOL | — | TRUE when light curtain zone is interrupted |
| Safety_Gate_Open | BOOL | — | TRUE when any interlocked safety gate is open |
| Safety_Reset_Required | BOOL | — | TRUE when a safety reset is required before restart |
| SIL_Watchdog_OK | BOOL | — | TRUE when safety PLC watchdog is healthy |

5.6  Site1_Inspection — Siemens S7-1200 Inspection and Sorting

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Inspection_Parts_Per_Min | REAL | — | Current inspection throughput rate |
| Inspection_Pass_Count | DINT | Count | Parts passed inspection this shift |
| Inspection_Fail_Count | DINT | Count | Parts failed inspection this shift |
| Inspection_Good_Bin | DINT | Count | Good parts in current bin |
| Inspection_Reject_Bin | DINT | Count | Rejected parts in current bin |
| Inspection_Scrap_Bin | DINT | Count | Scrapped parts in current bin |
| Inspection_Lot_Status | STRING | — | Quality status of current lot — PASS / FAIL / REVIEW |
| Inspection_Batch_Complete | BOOL | — | TRUE one scan when inspected batch is complete |

5.7  Site1_Conveyor — Siemens S7-1200 Intra-Site Conveyor

The conveyor is a transport device. OEE is not calculated. Status, drive health, and throughput tags apply.

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Conveyor_Running | BOOL | — | TRUE when conveyor system is running |
| Conveyor_Stopped | BOOL | — | TRUE when conveyor is intentionally stopped |
| Conveyor_Faulted | BOOL | — | TRUE when a conveyor fault is active |
| Conveyor_Jam_Detected | BOOL | — | TRUE when jam condition is detected |
| Drive01_Running | BOOL | — | TRUE when Drive 01 is running |
| Drive01_Speed_RPM | REAL | RPM | Drive 01 actual speed |
| Drive01_Fault | BOOL | — | TRUE when Drive 01 has an active fault |
| Drive02_Running | BOOL | — | TRUE when Drive 02 is running |
| Drive02_Speed_RPM | REAL | RPM | Drive 02 actual speed |
| Drive02_Fault | BOOL | — | TRUE when Drive 02 has an active fault |
| Conveyor_Parts_Shift | DINT | Count | Parts conveyed this shift |
| Conveyor_Fault_Count_Shift | INT | Count | Fault events this shift |

5.8  Site1_Palletizing — Micro850 Palletizer and Bulk Bin Staging

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Pallet_Bin_Fill_Pct | REAL | Pct | Current bulk bin fill level |
| Pallet_Bin_Count_Shift | DINT | Count | Bins completed this shift |
| Pallet_Parts_Per_Bin | DINT | Count | Parts per bin — configured value |
| Pallet_Complete | BOOL | — | TRUE one scan when pallet is complete |
| Pallet_Bins_Staged | INT | Count | Bins staged and ready for Site 2 transfer |
| Lot_Number | STRING | — | Active lot number — YYYYMMDD-XXXX format |
| Lot_Part_Count | DINT | Count | Total parts in active lot |
| Lot_Complete | BOOL | — | TRUE one scan when lot is complete and ready for transfer |
| Transfer_Request | BOOL | — | TRUE when Site 2 transfer is requested — published to UNS |

5.9  Site1_Supervisory — groov EPIC-PR1 Site 1 Command

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Line_Running | BOOL | — | TRUE when production line is in active production |
| Line_Stopped | BOOL | — | TRUE when line is intentionally stopped |
| Line_Faulted | BOOL | — | TRUE when any machine on the line has an active fault |
| Line_OEE_Pct | REAL | Pct | Calculated line-level OEE |
| Line_Availability_Pct | REAL | Pct | Line-level availability |
| Line_Performance_Pct | REAL | Pct | Line-level performance |
| Line_Quality_Pct | REAL | Pct | Line-level quality |
| Line_Parts_Hour | REAL | Pph | Current line throughput rate |
| Line_Parts_Shift | DINT | Count | Total parts produced this shift — all machines |
| Line_Parts_Day | DINT | Count | Total parts produced today |
| Shift_Number | INT | — | Current shift number — 1, 2, or 3 |
| Shift_Parts_Target | DINT | Count | Shift production target |
| Shift_Parts_Actual | DINT | Count | Shift production actual |
| Active_Alarm_Count | INT | Count | Number of active alarms across all machines |
| Line_Start_Cmd | BOOL | — | Operator command — start production line |
| Line_Stop_Cmd | BOOL | — | Operator command — stop production line |
| Line_Reset_Cmd | BOOL | — | Operator command — reset all faults |

5.10  Site2_Packaging — Wago PFC200 Packaging, Warehousing, and Shipping

| Tag Name | Type | Unit | Description |
| --- | --- | --- | --- |
| Pkg_Machine_Running | BOOL | — | TRUE when packaging system is running |
| Pkg_Machine_Stopped | BOOL | — | TRUE when packaging system is stopped |
| Pkg_Machine_Faulted | BOOL | — | TRUE when packaging system has an active fault |
| Pkg_Box_Fill_Pct | REAL | Pct | Current box fill level — parts vs target |
| Pkg_Box_Weight_Lb | REAL | Lb | Simulated box weight at current fill level |
| Pkg_Parts_Per_Box | INT | Count | Configured parts per box |
| Pkg_Box_Complete | BOOL | — | TRUE one scan when box is packed and sealed |
| Pkg_Boxes_Shift | DINT | Count | Boxes completed this shift |
| Pkg_Lot_Number_Active | STRING | — | Lot number currently being packed — received from UNS |
| Whse_Inventory_Total | INT | Count | Total boxes currently in warehouse |
| Whse_Slots_Occupied | INT | Count | Number of warehouse slots in use |
| Whse_Slots_Available | INT | Count | Number of warehouse slots available |
| Ship_Shipments_Today | INT | Count | Shipments completed today |
| Ship_Last_Lot | STRING | — | Lot number of most recent shipment |
| Ship_Last_Qty | INT | Count | Box quantity of most recent shipment |
| Ship_Last_Timestamp | DINT | — | Timestamp of most recent shipment event |
| Recon_Site1_Count | DINT | Count | Parts received count reported by Site 1 |
| Recon_Site2_Count | DINT | Count | Parts received count verified at Site 2 |
| Recon_Variance | DINT | Count | Count discrepancy — Site1 minus Site2 |
| Recon_Status_OK | BOOL | — | TRUE when variance is within acceptable tolerance |

6.  Shift Reset Behavior

Tags designated as shift-accumulating reset at the start of each shift. Cumulative tags never reset. This distinction is encoded in the tag name suffix.

| Tag Name Pattern | Reset Behavior | Example |
| --- | --- | --- |
| _Shift suffix | Resets at shift change | Parts_Shift, Reject_Count_Shift |
| _Total suffix | Never resets | Parts_Total |
| _Today suffix | Resets at midnight | Ship_Shipments_Today |
| No time suffix | Current value — no accumulation | Machine_Running, OEE_Pct |

7.  Worked Examples — Full Tag Names

-- Machine status Site1_Stamping_Machine_Running Site1_Wash_Machine_Faulted Site2_Packaging_Pkg_Machine_Running -- OEE Site1_Stamping_OEE_Pct Site1_Forming_Availability_Pct Site1_Inspection_Quality_Pct -- Production counts Site1_Stamping_Parts_Shift Site1_Trimming_Parts_Total Site1_Inspection_Inspection_Pass_Count -- Process values Site1_Wash_Wash_Temp_F Site1_Forming_Forming_Die_Temp_F Site1_Wash_Wash_Water_Pressure_PSI -- Fault and downtime Site1_Stamping_Fault_Active Site1_Stamping_Fault_Code Site1_Wash_Downtime_Duration_Sec -- Cross-site data handshake Site1_Palletizing_Lot_Number (origin — Micro850) Site1_Palletizing_Transfer_Request (trigger — published to UNS) Site2_Packaging_Pkg_Lot_Number_Active (received — Wago PFC200) Site2_Packaging_Recon_Variance (reconciliation result)

| DOCUMENT INFORMATION |  |
| --- | --- |
| Document Title | UNS Subscription Register — Loogootee Mfg. Training Simulation |
| Prepared By | Jeremy Field — Senior Automation Technician |
| Version | Rev 0.1 — Initial Issue |
| Date | March 2026 |
| Status | Training Simulation — Not for Distribution |
| Repository | github.com/JeremySField/loogootee-mfg-training-simulation |
| Applies To | All PLC programs, UNS topics, historian tags, and dashboard metrics across this engagement |

Loogootee Mfg., Inc.  ·  Training Simulation — Not for Distribution